VW-SS-01(For VW Series) 





Model No.:  VW-SS-01
Car CAN BUS Interface
1、 Manufacturer Models
 
	VOLKSWAGEN
	PASSAT CC
	SKODA
	OCTAVIA
	OPEL
	ASTRA

	
	TOURAN
	
	
	
	

	
	TIGUAN
	
	FABIA
	
	

	
	GOLF V
	
	
	
	CORSA

	
	GOLF VI
	
	SUPERB
	
	

	
	JETTA 3
	
	
	
	ANTARA

	
	SAGITAR
	
	
	
	

	
	MAGOTAN
	
	
	
	


 

2、 Connector definition（ Odd number at the bottom, Even number at the top.）
[image: image1.jpg]



DEFINITION of 16 PINS
	PIN
	Definition
	PIN
	Definition

	1
	BATT INPUT
	9
	SPEED SIGNAL OUTPUT

	2
	GND
	10
	CTRL_IN（alternatively）

	3
	CAN_L INPUT
	11
	IR_IN （alternatively）

	4
	CAN_H INPUT
	12
	+5V（alternatively）

	5
	Ignition (ACC) OUTPUT
	13
	IR_OUT

	6
	ILL OUTPUT
	14
	RXD（alternatively）

	7
	REVERSE OUTPUT
	15
	TXD（alternatively）

	8
	PARK OUTPUT
	16
	TEST（alternatively）


 

3、Functions:
1) . Ignition function

2)  Illumination function

3)  Reverse Signal

4)  Park Brake function

5)  Speed information

6)  Control of open / close the door

7)  Steering wheel control of car stereo

 

 

AN is the abbreviated name for Controller Area Network.
CAN Bus wiring was originally designed by Bosch to link electrical control systems together to form a network of information around the vehicle. CAN Bus is replacing the conventional wiring as it is less bulky and offers manufacturers a cost saving on running copper wire throughout a vehicle, however this does make it increasingly difficult to retrofit products to modern vehicles.
Functions:
1) . Ignitionfunction
2) . Illumination function
3) . Reverse Signal
4) . Park Brake function
5) . Speed information
6) . Control of open / close the door
7) . Steering wheel control of car stereo
8) . Display of the original dashboard information ( The information of DVD Navigation system) and operation of steering wheel menu.
9) . Original Audible Alarm
10) . Original stereo information
11) . Amplifier control of original stereo system
12) . Original air conditioner information display
13) . Keep the original rear zone audio control
14) . Set up the information of car
